Neuropixels Data Processing Pipeline

(Python based)

Neural Data

- Filter ap data and extract ttl times (CatGt)

In CatGt folder edit and run batch file (runit.bat). Specify in batch file what needs to be done. Use CatGt_runit_example file contents as a guide to fill in new information.

G should match recording, normally g0, but change if not.

Generally, apply bandpass filter (butter,12,300,9000) and glbcar.

Make sure right analog channels are chosen (playback ttl, stim ttl). Ttl in ap stream/obx is extracted by default.

Example of batch file:

Parent folder and folder containing recording files

set LOCALARGS=-dir=M:\ -run=test_Stim ^

Gate and trigger

-g=0 -t=0,0 -prb_fld ^

Neural channels extracted, filtered, and car applied

-ap -prb=0 -apfilter=butter,12,300,9000 -gbldmx -gfix=0.40,0.10,0.02 ^

One box analog channel 1 (mirophone) and 4 (stim pulse ttl) extracted

-ob -obx=0 -xa=1,0,0,1,0,0 -xa=1,0,4,1,0,0 ^ ^

Destination folder

-dest=M:\test_Stim_g0\test_Stim_g0_imec0 ^

- Spike sorting: Run kilosort 4

First use SGLXMetaToCoords.mat script to generate a mat file specifying the channel map for the current recording. Select output type 1. Output is a mat file with

Then, open Kilosort GUI: a) in anaconda cmd write conda activate Kilosort, b) python -m kilosort

If GUI is not opening run “python -m kilosort --reset”. Kilosort always tries to retrieve previous dataset, sometimes cannot find it and wont open.

In GUI select ap file to sort. Usually CatGt filtered file.

Probe Layout: For Neuropixels 1, just pick corresponding map from list. For Npx2 specify the channel map with the mat file previously generated.

Sampling frequency: If headtage was calibrated, change sampling rate to that one (in meta file). In Kilosort 4 there is no need to have npy-matlab library in Kilosort folder, it comes with the installation (python based).

Default parameters are:

Data dtype: int16, Number of channels: 385, Sampling frequency: 30000, Batch size: 60000                    Nblocks: 1, Th (universal): 9, Th (learned): 8, Tmin: 0, Tmax: inf

Kilosort 4 parameters: https://kilosort.readthedocs.io/en/latest/parameters.html

- Do first run of quality metrics

This helps assessing units when using phy to curate.

- Manual curation using phy

Use this step either to merge and split clusters or to remove noise clusters (for this label all clusters).

To use phy open anaconda prompt, then:

conda activate phy2

pushd path to kilosort output

phy template-gui params.py

General procedure when curating:

Step 1 - select a cluster from ClusterView

Step 2 – check waveform similarity

Step 2.1 – check Autocorrelogram (refractory period) and amplitude

Step 3 - assess cross correlograms – refractory period, check channel, same (triangulation)?

Step 4 - consider FeatureView and AmplitudeView – check stability on amplitude and normality

Step 5 – if available, check quality metrics values of both clusters being compared

Step 6 – classify and label

Step 7 - save your progress

“””Be careful with inconsistencies in number of clusters in cluster info and group”””

Files updated by phy: Cluster_Amplitude.tsv, cluster_ContamPct.tsv, cluster_group.tsv (ony clusters labeled in phy), cluster_info.tsv, cluster_KSLabel.tsv, spike_clusters.npy

Files not updated by phy: clus_Table.npy (necessary if c_waves si used)

If phy is not opening correctly because temp_wh.dat file was originally run in a different folder than the current one, change path in params.py file.

blocks = 1;

ibi = 10;

repetitions = 75;

iri = 2;

catch_trials = 0;

catch_duration = 1.100000;

sampling_rate = 44100;

Stim/Playback Data

- Get stimuli durations

Stim pulses: Check notes and gather information on what pulses were presented and how many times.

Playbacks: Run playback_edges.m to detect onset and offset of playbacks (it generates csv file). Then, check if borders are correctly labelled, otherwise adjust them manually and overwrite values in corresponding csv file. Use Audacity or Adobe Audition to visualize the wav file and adjust borders when needed, then update playback edges output file.

Once durations are correct, create a text file with 2 columns, 1 for labels and 1 for stimuli durations.

- Get event times (from ttls)

Typically, a ttl is used in at the onset of a stimulus (plb/stim pulses). CatGt extracts these times in a .txt file. Alternatively, use script extract_ttl_events.ipynb.

It is important to confirm that the number of ttl events obtained corresponds to the actual times a stimulus was used. For this, the same script extract_ttl_events can be used.

To crosscheck if a given ttl corresponds to a real playback use SpikeGlx File Viewer or the exported wav file from the channel of interest.

Once .txt file is curated, the script will create a .txt file with the onsets of all stimuli presented. All event times must be in ascending order.

- Synchronize stimuli times with neural data

Use TPrime to translate event times so that they are aligned with spike times. Times must always be written in ascending order. Use TPrime_runit_example file contents as a guide to fill in new information. Run batch file. Output will be a new .txt file with aligned times and ending in _tprime.

Parameters:

toStream: path and .txt file with sync ttls from ap.

fromStream: path and .txt file with sync ttls from ni or obx.

events: path and .txt file with event times, and path and file name of new _tprime document.

Notes: When writing events in TPrime specify input and output separated by a comma (,).

- Assemble summary csv with event times and labels

Use script create_event_csv.ipynb to generate a csv file with event onset times, tprime times and corresponding labels. Follow the instructions depending on the event type in use:

Stim pulses: Use notes from Stim_experiment_Form_M9.csv to specify in the script how many times each stimulus was presented. More details in script.

Playbacks: Confirm the existence of a) “Playbacks” folder and that it contains all playbacks used for current recording (experiment), b) Metadata file describing how playbacks were presented, and c) Csv file specifying the order in which playbacks were presented during experiment and their labels. If the csv file is not in the same location as the ttl_event times extracted before, make a copy there.

- Sanity check

To confirm that ttls really correspond to stimuli onsets, run ttl_sanity_check.ipynb. It will plot all ttl events plus a selected Neuropixels channel.

Analyzing Data

Run all_clusters_psth_variables.mlx